# A&E Specifications Rev 0815 2M Standard IPCAM

# Surveon Smart Megapixel Recorder Series

## Description

2M Standard IPCAM are professional network cameras that use Internet Protocol (IP) to transmit video streams and control signals over networks. Capable of operating over both LANs and WANs, they provide a complete budget-conscious remote surveillance solution that are ultra clear and highly integrated. 2M Standard IPCAM combine a user-friendly interface and simplified installation with a powerful feature set to provide users an easy upgrade path to new digital surveillance system in a virtual environment. These highlights make 2M Standard IPCAM ideal choices for environments that require remote surveillance or video transmission.

## Bid Specification

### Model Numbers

The 2M Standard IPCAM shall include the following models:

| CAM2301A | Full HD D/N Network Camera |
| --- | --- |
| CAM3351 | Full HD D/N Bullet Network Camera |
| CAM4311 | 2M D/N IP Fixed Dome |
| CAM4321 | Full HD Day & Night Fixed Dome Network Camera |

The camera shall utilize the following image sensor:

1/2.7" 2 megapixel progressive scan CMOS

The camera shall have the following lens:

| CAM2301A | Changeable (CS/C mount) |
| --- | --- |
| CAM3351 | 4.2mm, F1.6 |
| CAM4311 | 4.0 mm, F2.0 |
| CAM4321 | 3.3 - 12 mm varifocal lens, F1.4 |

The camera shall have the SNR of 48dB.

The camera shall be equipped with WDR functionalities.

The camera shall be equipped with Day/Night ICR functionalities.

The camera shall have IR IED:

| CAM2301A | N/A |
| --- | --- |
| CAM3351 | Yes (15M) |
| CAM4311 | Yes (10M) |
| CAM4321 | Yes (20M) |

The camera shall support the minimum illumination of :

0.01 Lux @ F1.2 (B/W)

0.1 Lux @ F1.2 (Color)

The camera shall utilize DC-iris control:

| CAM2301A | Yes |
| --- | --- |
| CAM3351 | N/A |
| CAM4311 | N/A |
| CAM4321 | Yes |

The shutter time of the camera ranges from 1/30 ~ 1/50,000 s.

The camera shall have the following viewing angle:

| CAM2301A | N/A |
| --- | --- |
| CAM3351 | Diagonal: 64.0° Horizontal: 56.0° Vertical: 32.0° |
| CAM4311 | Diagonal: 101.7°° Horizontal: 87.7°° Vertical: 48° |
| CAM4321 | Diagonal: 99°~37.3° Horizontal: 79.3°~29.8° Vertical: 59.5°~22.4° |

The camera shall have triple standard compression support with simultaneous streaming of H.264, MPEG-4, and MJPEG formats.

The camera shall output at a maximum resolution of 1920 x 1080.

It shall be possible to program the camera to output a variety of resolution images:

25 fps at 1080P (1920 x 1080)

30 fps at SXGA (1280 x 1024)

30 fps at HD720 (1280 x 720)

30 fps at D1 (720 x 480)

30 fps at VGA (640 x 480)

30 fps at QVGA (320 x 240)

The camera shall feature the following video controls:

AGC (Auto Gain Control), AWB (Auto White Balance), AES (Auto Electronic Shutter), BLC (Back Light Compensation), Image Adjustment

The camera shall have the bit rate control: 64K ~ 6Mbps, VBR, CBR, controller frame rate and quality.

The camera shall feature the following video intelligence:

Motion detection, Tampering Detection (blocked, redirected, defocused, or spray-painted)

The camera shall support video jack:

| CAM2301A | Yes (BNC) |
| --- | --- |
| CAM3351 | N/A |
| CAM4311 | N/A |
| CAM4321 | Yes (BNC) |

The camera shall support audio compression at 32KHz, ADPCM:

| CAM2301A | Yes |
| --- | --- |
| CAM3351 | N/A |
| CAM4311 | N/A |
| CAM4321 | N/A |

The camera shall have the audio input/output at 3.5mm phone jack:

| CAM2301A | Yes |
| --- | --- |
| CAM3351 | N/A |
| CAM4311 | N/A |
| CAM4321 | N/A |

The camera shall be with alarm in/out at 1/1, terminal block:

| CAM2301A | Yes |
| --- | --- |
| CAM3351 | N/A |
| CAM4311 | N/A |
| CAM4321 | N/A |

The camera shall support video buffer at 5 second pre-alarm, 30 second post-alarm.

The camera shall allow these event actions: Send snapshot or video clip by FTP or email, record to NAS, record to local storage, trigger DO.

The camera shall support these protocols: IPv4, ARP, TCP, UDP, ICMP, DHCP, NTP, DDNS, SMTP, FTP, HTTP, CIFS, PPPoE, UPnP, RTP, RTSP, RTCP, 3GPP.

The camera shall be equipped with 10/100 Base-T / RJ45 connector.

The camera shall support the following SD card: Class2/Class 4/Class 6.

The camera shall utilize the RS-485:1 (2 pin on terminal block):

| CAM2301A | Yes |
| --- | --- |
| CAM3351 | N/A |
| CAM4311 | N/A |
| CAM4321 | N/A |

The camera shall utilize Surveon SDK 2.0.

The camera shall support these OS: Microsoft Windows XP/Vista/7.

The camera shall support Microsoft IE 6.0 or above.

The camera shall be equipped with the following software: Surveon VMS 2.4.7

The camera’s ambient temperature shall be as follows:

Operation: -10~50°C (14~122°F)

Storage: -30~60°C (-22~140°F)

The camera’s ambient humidity shall be 5~90%.

The camera’s primary power source shall be Power over Ethernet (PoE) complying with the IEEE 802.3af standard or 12V DC power supply.

The camera shall follow the following power consumption:

| CAM2301A | Max. 7W |
| --- | --- |
| CAM3351 | Max. 7W (w/o Heater ) |
| CAM4311 | Max. 7.5W |
| CAM4321 |  |

The camera shall have total unit dimensions:

| CAM2301A | 74.95mm x 59.3mm x 153.5mm (2.95" x 2.34" x 6.04") |
| --- | --- |
| CAM3351 | 75mm x 76mm x 172mm (2.46” x 2.49” x 5.64”) |
| CAM4311 | ø128.8mm x 98.6mm (H) ø5.07” x 3.88”(H) |
| CAM4321 | ø128.8mm x 115mm (H) ø5.07” x 4.52" (H) |

The camera shall have a total unit weight as follows:

| CAM2301A | Net: 435g (0.96 lb) Gross: 1085g (2.4 lb) |
| --- | --- |
| CAM3351 | Net: 667g (1.47 lb.) Gross:1,014g (2.24 lb.) |
| CAM4311 | Net: 620g (1.37lb.) Gross: 1290g (2.85lb.) |
| CAM4321 | Net: 700g (1.54lb.) Gross: 1370g (3.00lb.) |

The camera shall be FCC Part 15, Class A, CE and IP66 compliant:

| CAM2301A | Safety: LVD EMC: FCC, CE |
| --- | --- |
| CAM3351 | Safety: LVD EMC: FCC, CE IP66 |
| CAM4311 | Safety: LVD EMC: FCC, CE |
| CAM4321 |  |

### Related Documentation

- CAM2xxx/3xxx/4xxx Series User Manual

- CAM2xxx/3xxx/4xxx Series Quick Installation Guide

## 3.0 Warranty

Surveon warrants to the direct customer that its product will be free from defects in material and workmanship for a period of 3 years from the date of delivery of the product, excluding batteries which Surveon warrants will be free from defects in material and workmanship, under normal use and service, for a period of 1 year. Surveon shall service all products as follows:

  - Surveon shall, at its option and expense, repair or replace the defective products. Replacement, if required, shall be with new, comparable, or reconditioned products.

  - Customer shall ship defective products freight prepaid to Surveon. Surveon shall ship the repaired/replacement products freight prepaid to the customer. If the customer has a pending delivery, the repaired/replacement products shall be shipped together with the customer's other order. Otherwise, it will be shipped separately. Both parties agree to use the same means of shipping.

  - Defective products or parts, if replaced, automatically become the property of Surveon Technology Inc.

  - All products repaired or replaced under warranty shall carry a warranty equal to the period of time remaining on the warranty based on the original delivery date.

  - Service response time may vary from different regions. Please contact your local representative for more information.