A&E Specifications Rev 051911 CAM4220

# CAM4220 Megapixel Color Camera

# Bid-Spec

## 1.0 Description

CAM4220 is a professional IP network camera utilizing Internet Protocol (IP) to transmit video streams and control signals over networks. Capable of operating over both LANs and WANs, CAM4220 provides a complete budget-conscious remote surveillance solution that is ultra clear and highly integrated. The CAM4220 combines a user-friendly interface and simplified installation with a powerful feature set, including Motion Detection and Tampering Detection to provide users an easy upgrade path to new digital surveillance system in a virtual environment. These highlights make CAM4220 an ideal choice for environments that require remote surveillance or video transmission.

## 2.0 Bid Specification

- The camera shall utilize a high sensitivity 1.3 Megapixel CMOS sensor with 1/3” optical format.

- The camera shall have 2.8 - 11 mm varifocal lens, with maximum aperture of F1.4.

- The camera shall have Day/Night ICR capabilities.

- The camera shall have built-in IR illumination, effective up to 30m.

- The camera shall have a dome enclosure.

- The camera shall be H.264 (MPEG4, Part 10) compliant.

- The camera shall have triple standard compression support with simultaneous streaming of H.264, MPEG-4, and MJPEG formats.

- The camera shall have multi-streaming support of up to 2 non-identical concurrent streams (different frame rate, bit rate, resolution, quality, and compression format).

- The camera shall have bit rate control for full view, multi view, and archiving.

- The camera’s bit rate control shall be selectable from 32 Kbps to 10 Mbps for each independent stream.

- The camera shall have Real Time Streaming Protocol (RTSP) support allowing for compatibility with media players such as RealPlayer, Apple QuickTime, VLC Player and others.

- The camera’s H.264 implementation shall maintain full real time video frame rates.

- The camera shall output at a maximum resolution of 1280(H) x 1024(V) pixels at a maximum frame rate of 15 frames per second (FPS).

- It shall be possible to program the camera to output a variety of lower resolution images. For example, 1280(H) x 720(V) pixels at 15 FPS, 640(H) x 480(V) pixels at 30 FPS, or 320 (H) x 240 (V) at 30 FPS.

- The camera shall be equipped with a 100 Mbps LAN connector.

- The camera shall support a minimum HTTP, and RTSP/RTP network protocols.

- The camera shall feature precise motion detection with a minimum of 3 detection zones.

- The camera shall feature automatic exposure, automatic white balance, shutter speed control, programmable brightness, saturation, contrast, instantaneous digital zoom, and electronic image mirror and flip.

- The camera shall incorporate necessary algorithms and circuits to detect motion in low light with clarity.

- The camera shall support a minimum illumination of 0 Lux.

- The camera’s primary power source shall be Power over Ethernet (PoE) complying with the IEEE 802.3af standard or 12V DC power supply.

- The camera’s operating ambient temperature shall be -10~50°C (14~122°F) for warm start conditions.

- The camera shall be FCC Part 15, Class A and CE compliant.

- The camera shall have total unit dimensions of ø128.8mm x 115 mm(H) ø5.07” x 4.52”(H).

- The camera shall have a total unit weight of: Net: 700g (1.54 lb).

# Quick-Spec

## 3.0 Minimum Performance Specification

Megapixel camera must meet the following operating requirements

### Imaging

- 1.3 megapixel CMOS image sensor

- 1280x1024 pixel array

- 1/3” optical format

- Bayer mosaic RGB filter

- Minimum illumination of: 0 Lux (IR Illuminators on)

### Full Field of View (FOV) Resolutions

- 1280x1024 1.3 MP

### Cropped Field of View Resolutions

- 1280x720 HDTV-720p

- 640x480 VGA

- 320x240 SIF

### Data Transmission

- Data rate

  - Bit rate control available for full view, multi view, and archive

  - Bit rate control from 32 Kbps to 10 Mbps

  - Video frame rate up to 15fps @ 1280x1024

- Compression type

  - H.264 (MPEG4, Part 10)

  - Motion JPEG

  - MPEG-4

- HTTP, RTSP image transmission protocols

- 100Base-T Ethernet Network Interface

### Programmability

- Auto Exposure (AE) and Gain Control (AGC)

- Programmable backlight compensation

- Auto white balance

- Electronic image flip - 180 degree rotation

- Programmable shutter speed to help control motion blur

- Proprietary noise cancellation

- Programmable resolution, brightness, saturation and contrast

- Bandwidth and storage savings by running at 1/4 resolution

### Electrical

- Power over Ethernet (PoE): PoE 802.3af

- DC input: auxiliary 12 V DC

- Power consumption 7.5 Watts maximum

### Regulatory Approvals

- FCC, Class A

- CE and RoHS compliant

### Environmental

- Operating temperature: -10~ 50°C (14~122°F)

- Storage temperature: -30~60°C (-22~140°F)

- Humidity: 5~90% (non condensing)

### Mechanical

- Die-cast aluminum chassis

- Total unit dimensions: ø128.8mm x 115mm(H) ø5.07” x 4.52”(H)

- 700g (1.54 lb)

### Lens

- 2.8 - 11 mm varifocal lens, F1.4

### Related Documentation

- CAM4220 Network Camera User Manual

- Network Camera SDK HTTP/RTSP, ActiveX and DLL control editions

- CAM4220 Network Camera Hardware Manual

## 4.0 Model Numbers

The camera shall be Surveon model CAM4220, 1.3 megapixel network camera.

## 5.0 Warranty

Minimum 3 year parts and labor