A&E Specifications Rev 052011 NVR500

# NVR500 Megapixel Network Video Recorder

# Bid Specification

## 1.0 Description

The Surveon NVR500 is a highly integrated 8 channel network video recorder featuring a virus free, installation free embedded design that is fully burn-in-tested. The megapixel-optimized NVR500 includes a function rich software suite, which is scalable with Surveon’s free CMS package. These features allow the NVR500 to provide a worry-free, low-overhead system operation aimed at smaller scale installations in diverse applications such as retail, school and hospitality.

## 2.0 Bid Specification

- The NVR shall support up to 8 network cameras.

- The NVR shall have a capacity of up to 3 SATA hard-drives.

- The NVR shall have a set of standard I/O ports, including:

  - 1 x Gigabit Ethernet Port

  - 1 x VGA D-Sub Port

  - 1 x Audio Line-in Port

  - 1 x Audio Line-out Port

  - 2 x USB 2.0 Ports

  - 1 x RS232 Port

  - 2 x PS2 Ports

- The NVR shall have external LED indicators for power, recording, network, and error statuses.

- The NVR shall have an embedded operating system.

- The NVR shall include intelligent Video Management Software.

- The NVR’s primary power source shall be 115 V AC @ 6 A / 240 V AC @ 3 A.

- The NVR’s operating ambient temperature shall be 10~35°C (50~95 °F).

- The NVR shall be operational in a humidity range within 5~95 % (non-condensing).

- The NVR shall be FCC Class B, CE, UL60959, IEC60950, BSMI, and CCC compliant.

- The NVR shall have total unit dimensions of 40.42 x 34.02 x 9.71cm (15.91"x 13.39"x 3.82").

- The NVR shall have a total unit weight of 11.36kg (26 lbs).

# NVR Specifications

## 3.0 Minimum Performance Specification

The NVR must meet the following operating requirements

### System

- Up to 8 Network Cameras

### Storage

- HDD Capacity 3 x SATA

### I/O Interface

- 1 x Gigabit RJ-45

- 1 x D-Sub

- 1 x Audio Line-in

- 1 x Audio Line-out

- 2 x USB 2.0

- 1 x RS232

- 2 x PS2

### LED Indicators

- Power

- Recording

- Network

- Error

### Electrical

- Input voltage : 115 V AC @ 6 A / 240 V AC @ 3 A

- Frequency: 47 to 63 Hz

- Power: 250 watt

### Environmental

- Temperature:

  - Operating: 10~35°C (50~95°F)

  - Non-operating: -40~70°C (-40~158°F)

- Humidity:

  - Operating: 5~95% (non-condensing)

  - Non-operating: 5~95% (non-condensing)

### Mechanical

- Total unit dimensions: 40.42 x 34.02 x 9.71 cm (15.91"x 13.39"x 3.82")

- 11.36kg (26 lbs)

### Regulatory Approvals

- FCC, Class B

- CE

- UL60959/IEC60950

- BSMI

- CCC

### Related Documentation

- NVR User Manual

## 4.0 Model Numbers

The NVR shall be Surveon model NVR500, 8 Channel Megapixel NVR.

## 5.0 Warranty

Minimum 3 year parts and labor