A&E Specifications Rev 051710 NVR2000

# NVR2000 Megapixel Network Video Recorder

# Bid Specification

## 1.0 Description

The Surveon NVR2000 is a powerful network video recorder supporting up to 64 channels of megapixel quality video. Featuring a virus free, installation free embedded design that is fully burn-in-tested, the NVR2000 eliminates compatibility issues while reducing maintenance overheads. In addition to this, the NVR2000 also features Surveon’s Clustered Video Storage Technology (CVST), enabling seamless online storage expansion and configuration, while reducing maintenance, provisioning, and equipment costs, making the NVR2000 a reliable cost-effective solution for medium to large sized surveillance needs.

## 2.0 Bid Specification

- The NVR shall support up to 64 network cameras.

- The NVR shall have a 1U rackmount chassis.

- The NVR shall have a capacity of up to 4 SATA-II hard-drives.

- The NVR shall support an option external RAID through SAS interface card.

- The NVR shall have a set of standard I/O ports, including:

  - 2 x Gigabit Ethernet Port

  - 1 x SAS Expansion Port

- The NVR shall have external LED indicators for overheat/fan failure, network port 1 and 2, hard drive access, power, and SATA drive statuses.

- The NVR shall have an embedded LINUX operating system.

- The NVR shall include intelligent Video Management Client capable of features such as E-Map and Virtual matrix.

- The NVR’s primary power source shall be 115 V AC @ 6 A / 240 V AC @ 3 A.

- The NVR’s operating ambient temperature shall be 10~35°C (50~95 °F).

- The NVR shall be operational in a humidity range within 5~95% (non-condensing).

- The NVR shall be FCC Class A, CE, CB, UL60959, IEC60950, and CCC compliant.

- The NVR shall have total unit dimensions of 50.4x 43.7x 4.3 cm (19.85" x 17.2" x 1.7").

- The NVR shall have a total unit weight of 17.3kg (38 lbs).

# NVR Specifications

## 3.0 Minimum Performance Specification

The NVR must meet the following operating requirements

### System

- Up to 64 Network Cameras

### Storage

- HDD Capacity 4x SATA-II

- Optional External SAS RAID

### I/O Interface

- 2 x Gigabit RJ-45

- 1 x SAS

### LED Indicators

- Overheat/Fan Fail

- Network 1

- Network 2

- Hard Disk

- Power

- SATA (Green/Red)

### Electrical

- Input voltage : 115 V AC @ 6 A / 240 V AC @ 3 A

- Frequency: 47 to 63 Hz

- Power: Single 250 watt

### Environmental

- Temperature:

  - Operating: 10~35°C (50~95°F)

  - Non-operating: -40~70°C (-40~158 °F)

- Humidity:

  - Operating: 5~95% (non-condensing)

  - Non-operating: 5~95% (non-condensing)

### Mechanical

- Total unit dimensions: 50.4x 43.7x 4.3 cm (19.85 " x 17.2 " x 1.7 ")

- 17.3kg (38 lbs)

### Regulatory Approvals

- FCC, Class A

- CE

- CB

- UL60959/IEC60950

- CCC

### Related Documentation

- NVR User Manual

## 4.0 Model Numbers

The NVR shall be Surveon model NVR2000, 64 Channel Megapixel NVR.

## 5.0 Warranty

Minimum 3 year parts and labor