| PROJECT NAME： GSe Pro 1000 series |  |  |  |  |  |  |  |  | Rev.：1.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION: (ref): GSe Pro 1004 series 1U4-bay RAID subsystem, Generic controller, SATA2, Intel Avoton C2538 GSe Pro 1008 series 2U/8-bay RAID subsystem, Generic controller, SATA2, Intel Avoton C2538 |  |  |  |  |  |  |  |  |  |  |  |  |
| PRODUCT MANAGER：Glenn.Lee |  |  |  |  | DATE：2017/06/06 |  |  |  |  |  |  |  |
| MODEL NAME / NUMBER : Gse Pro 1000 serial models Gse Pro 1004 + no optional host boards Gse Pro 1008 + no optional host boards |  |  |  |  |  |  |  |  |  |  |  |  |
| APPROVED： FW PM HW TM FW TM ME TM SW TM |  |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT ORIGINATOR |  |  |  |  | PM MANAGER |  |  |  |  |  |  |  |
| NOTIFY： |  |  |  |  |  |  |  |  |  |  |  |  |
| SALE |  | ### SOFTWARE | HARDWARE | MFG. MGR. |  | GEN.MGR. |  |  |  |  |  |  |
| ## Change History List |  |  |  |  |  |  |  |  |  |  |  |  |
| Rev | Update Reason |  |  |  |  |  | Updated Date |  |  | Updated By |  | Approved By |
| 1.0 | New Release |  |  |  |  |  | 2017/06/06 |  |  | Glenn.Lee |  |  |

Section 1: Overall view for whole product

| Category | Functional Specification | Remark |
| --- | --- | --- |
| Innovation | First IFT cloud gateway product |  |
| EonStor serial models products definition | Per C0 spec., EonStor GS serial models are: - GSe Pro 1004 - GSe Pro 1008 EonStor FS C0 MKT document: - \\server02\Marketing\Product Planning\C0\Unify storage\Cloud Gateway HDD Bundle policy: - Any number of "Drive + Plastic Tray" - HDD types and q’ty are defined by Planning Cloud Gateway Product feature: Please follow the latest GS product feature: Host Board Support types: No host board design GSe Pro 1000 series Performance requirement: Product Family; GSe Pro 1000. CPU; Avoton C2538. Drive side interface; SATA 6G. GSe File read (MB/s); 400. GSe File write (MB/s); 400. GSe Pro 1000 series JBOD Support Models and Max Drive Policy: N/A |  |
| DIMM Configuration Requirements | Default configuration: 4GBx1 Support DDR3 up to 32G (16Gx2) Optional configurations: 8G (4G x2), 16G (8Gx2), 32G (16Gx2) |  |
| Super Cap and SSD requirements | No Super Cap. |  |
| Push button requirement | For Reset CTRL to Restore default Status |  |
| LCD and LED Requirements | Only support LED PNL; HW design reserve LCD LED follows ES_LED_FP_v1.22 or later version |  |
| HDD Support requirements | Support mixing SATA 3G/6G drives on AVL Support SATA-SSD (SSD: follow HDD AVL plan) |  |
| Form-factor requirements | Follow Enhance 3US and 2US ME design. |  |
| RoHS requirement | RoHS complaint is a MUST from development stage |  |
| PSU Requirements | For 2U12/3U16 platform: 3Y-460W RAID Form-factor; Enclosure Name; Controller; PSU; Vendor. 1U4; 1Ux; Generic; 250 Watt; Delta. 2U8; 2Ux; Generic; 250 Watt; Delta. |  |
| Patent Concern | N.A. |  |
| MTBF | Controller: 500,000 hours Subsystem: 25,000 hours Power supplies: 250,000 hours Fans: 500,000 hours |  |
| EMC / Safety | Certification product names: should follow IFT business method of CTO EMC: BSMI, FCC / CE Class A (Class A is Acceptable). Safety: UL / cUL, CB, and ROSTEST (Russia Certificate) |  |
| Env. | - Thermal: System working temperature should follow specs. - Normal Operating: The temperature should be below 40C. - Abnormal Operating (FANx1 or PSUx1 Failure): The temperature should remain below 40C - Due to CPU spec, it cannot operate below 5C. Please test the system above 5C or equal to 5C to meet the CPU spec. - Thermal concern Super Cap the Spec of Super cap is -25C~60C, Therefore it is recommend that w/o Super cap installed the room temperature cannot over 40C and W super cap installed the room temperature cannot over 40C. |  |
| Interoperability Req. | Refer to TD-4-06 Interoperability Test item of later version for SxF |  |

Section 2: User interface and customer related information

| Category | Functional Specification | Remark |
| --- | --- | --- |
| Connectors of rear-side of CTRL | For Single CTRL - 1* mini USB connectors provide COM1 connectivity to HST. - 4* RJ-45 connectors (1 in 1) provide 1GB (Management?) function – on Main board - 2* USB 2.0 connector – on Main board - 2* USB 3.0 connector – on Main board |  |
| CTRL Drawing |  |  |
| LED indicator on CTRL | Please refer to ES_LED_FP_v1.22 or later version for detailed LED definitions Ctrl 1 LED in green/Amber, for Controller Status Ctrl 2 LED in amber, for Cache Dirty status Steady on; 1. Cache Memory is dirty or 2. Flushing data from FBM to drive (when power back). Blink; Writing cached data from cache memory to FBM* during power outage. Once writing is done, all LEDs are turned off.. Off; No data is in cache memory. Note : FBM : Flash Backup Module Ctrl 3 LEDs in green/amber to indicate CBM* Status Steady on in Green; CBM is ready which means BBU (Super Cap.) and SSD Card both are present. Blink in Green; BBU (Super Cap.) is charging. Steady on in Amber; 1. BBU (Super Cap.) or SSD Card or both failed 2. No BBU (Super Cap.) and/or SSD Card are found in Redundant models.. Off; BBU (Super Cap.) is not installed in Generic Models. Note : CBM: Cashe Backup Module Ctrl 4 LED in green, Fast Blink for Host Busy Restore Default Button: LED in GREEN, the default of NVRAM is restored Management Port *1 (1 in 1 connector): Speed LED on RJ-45 in green for 1G and blank for 10/100, Active LED Blink for LAN is active iSCSI 1G *4 ports (1 in 1 connector): Speed LED on 4x RJ-45 in green for 1G and blank for 10/100, Active LED Blink for LAN is active |  |

Section 3：CTRL PCBA and Host Board PCBA requirement

| Category | Functional Specification | Remark |
| --- | --- | --- |
| 1U4/2U8: RAID engine G Models CTRL PCBA (3822GSP04E and 3822GSP08E) | For GSe Pro 1004 Models CTRL : For 2U/3U serial RAID engines CTRL use the same PCBA (PCBA named as 3822GSP04E) For GSe Pro 1008 Models CTRL : For 2U/3U serial RAID engines CTRL use the same PCBA (PCBA named as 3822GSP08E) The design needs to keep the flexibility. |  |
| DRV backplanes | Different products use different Base-units w/ DRV BP PCBAs. And naming are listed as below: F/F; G/R; Base-unit; DRV BP PCB; DRV BP PCBA; Models. 1U4; G; ESS04G2B2Dx-0035; 9583ADP04G; 9583ADP04G; GSe Pro 1004. 2U8; G; ESS08G2B2Dx-0035; 9583ADP08G; 9583ADP08G; GSe Pro 1008. Note 1. - D means EonStor DS, - x=0 means no bundle HDD and x=P means half qty of bundle HDD (8 for 3U, and 6 for 2U) Please refer to ESS16F-X1840_FTS_V1.0 or later version for detail spec Please refer to \\Server02\pm\PCC infor system\A667_models_naming for Enclosure/PCBA naming |  |
| 1U4: other small boards | - Power board: 9583PWR1U 1U height, redundant PSU design. |  |
| 2U8: Other small boards | - Power board: 9583PWR2U 2U height, redundant PSU design. - Adaptor board: 9583ADP To access Power board and Backplane. |  |
| Cache Backup Module for Caching Data | R CTRLs are having Cache Backup Module for default. G CTRLs are option. |  |
| FW feature spec | Use FW v3.91 or later version (follow EonStor G6) |  |
| SW feature spec | Use SW v2.3.x or later version (follow EonStor G6) |  |

Appendix 1: CTRL Key parts and general information of each model

| Controller | DS RAID CTRL |
| --- | --- |
| 3822GSP04E | GSe Pro 1004 |
| 3822GSP08E | GSe Pro 1008 |
| Memory module per CTRL |  |
| DDRIII 4GB | Default, 4GB*1 |
| DDRIII 8GB | Optional, 4GB*2 |
| DDRIII 16GB | Optional, 8GB*2 |
| DDRIII 32GB | Optional, 16GB*2 |

Appendix 2: Spare parts and Accessories

| CTRL FRU P/N | GSe Pro 1004 | GSe Pro 1008 |
| --- | --- | --- |
| 3822GSP04E | V |  |
| 3822GSP08E |  | V |

| GSe Pro 1000 Series |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BP FRU P/N |  | GSe Pro 1004 |  | GSe Pro 1008 |  |
| 9583ADP04G |  | V |  |  |  |
| 9583ADP08G |  |  |  | V |  |
| Level-1 | Level-2 |  | FRU P/N |  | GSe Pro 1000 |
| Cables | Mini USB Cable |  | 9370ASCAB-0010 |  | Y |

### 2. Enclosure FRU

| Level-1 | FRU P/N | GSe Pro 1000 |
| --- | --- | --- |
| PSU | 9583CPSU-0010 | Y |
| Slide Rail | 9571CSLIDER-0010 | Y |
| Ear Rail | 9571CEAR-0010 | Y |

### Appendix 5: Content of Accessory Box (for ref.)

For GSe Pro 1004 series models

- One GUI CD pack, HW manual and operation manual inside

- One EonOne CD pack

- Two power cords

- One mini-USB to DB-9 cable

- 20 screws for HDD installation

- 16 screws for mounting enclosure to rack（M6 x4 and M5 x4 and #10-32 x4）

For GSe Pro 1008 series models

- One GUI CD pack, HW manual and operation manual inside

- One EonOne CD pack

- Two power cords

- One mini-USB to DB-9 cable

- 40 screws for HDD installation

- 16 screws for mounting enclosure to rack（M6 x4 and M5 x4 and #10-32 x4）

| Models | GSe Pro 1000 series |
| --- | --- |
| CPU | Intel CPU Avoton C2538 |
| RAID ENGINE | CPU/SW RAID |
| Memory socket | DDR3 x 2 sockets |
| DIMMs | 4GB DIMM x 1 (default) |
|  | (MAX 32GB per CTRL) |
| Flash ROM | 16M x 2 |
| NVRAM | FRAM FM21L16 (2Mbit) |
| Flash Module | m.2 64G MLC Flash |
| On Board LAN (1G) | RJ45 w/ 2 LED x 4 (88E1543) |
| M.2 connector | Yes x 1 |
| RS232 port | min USB |
| USB 2.0 | x2 |
| USB 3.0 | x2 |
| LED indicator | - Controller status |
|  | - CBM status |
|  | - Cache Dirty Status |
|  | - Host busy |
|  | - LAN Link |
|  | - LAN Active |
|  | - Restore Def |
|  | - 4*CH0 Link / Speed status (iSCSI 1G) |
|  | - 4*CH1 Link / Speed status (iSCSI 1G) |
| Push button | Yes (HW reserve only) |
| LCD/ LED PNL | LED support only |
| Buzzer | Yes |
| Driver Interface | SATA only |
| Drive chip | (1) Marvell 9235*1 (for GSe Pro 1008) |
|  | (2) Integrated SATA 3G port |
| Drive channel allocation | Please check the Block Diagram for details |
| Connector to BP | GSe Pro 1004: (1 x 5*6 pin connector + 1 x 9*6 pin “4 wall” Airmax) GSe Pro 1008: (1 x 5*6 pin connector + 2 x 9*6 pin “4 wall” Airmax) |
| I2C interface | YES |
| PCBA | 3822GSP04E (GSe Pro 1004) 3822GSP08E (GSe Pro 1008) |

Architecture for system block diagram

For GSe Pro 1004 series

GSe Pro 1008 series

ME Drawing

- Front view

1U4

2U8

- Rear view

1U4

2U8

- System

CTRL