| PROJECT NAME： GSe Pro 3008 series |  |  |  |  |  |  |  |  | Rev.：1.2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION: (ref): GSe Pro 3008 series 2U8-bay RAID subsystem, Generic controller, SATA, Single PSU, Intel Broadwell-DE 4-Core CPU D-1517 2U8-bay RAID subsystem, Generic controller, SATA, Redundant PSU, Intel Broadwell-DE 4-Core CPU D-1517 |  |  |  |  |  |  |  |  |  |  |  |  |
| PRODUCT MANAGER：Glenn.Lee |  |  |  |  | DATE：2017/07/07 |  |  |  |  |  |  |  |
| MODEL NAME / NUMBER : GSe Pro 3008 serial models GSe Pro 3008SP + dual optional host boards (4 in 1, iSCSI 10G (RJ45) , iSCSI 10G SFP+, SAS 12G, iSCSI 1G, Infiniband 56G x 2, iSCSI 40G x 2 , FC16 x2, FC16G x4, SAS 12G JBOD Expansions ) GSe Pro 3008RP + dual optional host boards (4 in 1, iSCSI 10G (RJ45) , iSCSI 10G SFP+, SAS 12G, iSCSI 1G, Infiniband 56G x 2, iSCSI 40G x 2 , FC16 x2, FC16G x4, SAS 12G JBOD Expansions ) |  |  |  |  |  |  |  |  |  |  |  |  |
| APPROVED： FW PM HW TM FW TM ME TM SW TM |  |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT ORIGINATOR |  |  |  |  | PM MANAGER |  |  |  |  |  |  |  |
| NOTIFY： |  |  |  |  |  |  |  |  |  |  |  |  |
| SALE |  | ### SOFTWARE | HARDWARE | MFG. MGR. |  | GEN.MGR. |  |  |  |  |  |  |
| ## Change History List |  |  |  |  |  |  |  |  |  |  |  |  |
| Rev | Update Reason |  |  |  |  |  | Updated Date |  |  | Updated By |  | Approved By |
| 1.0 | New Release |  |  |  |  |  | 2017/06/06 |  |  | Glenn.Lee |  |  |
| 1.1 | Update Single PSU models |  |  |  |  |  | 2017/07/07 |  |  | Glenn.Lee |  |  |
| 1.2 | Remove Single PSU model |  |  |  |  |  | 2017/10/23 |  |  | Glenn.Lee |  |  |

Section 1: Overall view for whole product

| Category | Functional Specification | Remark |
| --- | --- | --- |
| Innovation | First IFT cloud gateway product |  |
| EonStor serial models products definition | Per C0 spec., EonStor GS serial models are: - GSe Pro 3008SP - GSe Pro 3008RP EonStor FS C0 MKT document: - \\server02\Marketing\Product Planning\C0\Unify storage\Cloud Gateway HDD Bundle policy: - Any number of "Drive + Plastic Tray" - HDD types and q’ty are defined by Planning Cloud Gateway Product feature: Please follow the latest GS product feature: Host Board Support types: Models; Host board types and ports; Qty per CTRL. GSe Pro 3000 series; Converged host board (4 in 1); X 2. iSCSI 10G (RJ45) x 2; X 2. iSCSI 10G SFP+ x 2; X 2. SAS 12G x 2; X 2. iSCSI 1G x 4; X 2. FC16G x 2; X 2. FC16G x 4; X 2. iSCSI 40G QSFP+ x 2; X 2. Infiniband 56G QSFP+ x 2; X 2. SAS 12G x 2 for JBOD Expansions; X 1. GSe Pro 3000 series Performance requirement: Product Family; GSe Pro 3008. CPU; BDE D-1517. Drive side interface; SATA 6G. GSe File read (MB/s); 400. GSe File write (MB/s); 400. GSe Pro 3000 series JBOD Support Models and Max Drive Policy: RAID Model Name; Support JBOD Models; Max Drive Number. GSe Pro 3008; JB 3012A JB 3060/3060L; 428 HDD. GSe Pro 3000 series JBOD Support Configurations and Max Drive Policy: RAID Form-factor; Recommended JBOD Configuration; Total Number of Disks. 2U8; RAID 3U12 x1 + JBOD 3U12 x 14; 176. RAID 3U12 x1 + JBOD 4U60 x 7; 428. GSe Pro 3000 Host board configurations list. Host Board 1; Host Board 2. Converged host board (4 in 1); None. iSCSI 10G (RJ45) x 2; None. iSCSI 10G SFP+ x 2; None. SAS 12G x 2; None. iSCSI 1G x 4; None. FC16G x 2; None. FC16G x 4; None. iSCSI 40G QSFP+ x 2; None. Infiniband 56G QSFP+ x 2; None. Converged host board (4 in 1); Converged host board (4 in 1). iSCSI 10G (RJ45) x 2; iSCSI 10G (RJ45) x 2. iSCSI 10G SFP+ x 2; iSCSI 10G SFP+ x 2. SAS 12G x 2; SAS 12G x 2. iSCSI 1G x 4; iSCSI 1G x 4. FC16G x 2; FC16G x 2. FC16G x 4; FC16G x 4. iSCSI 40G QSFP+ x 2; iSCSI 40G QSFP+ x 2. Infiniband 56G QSFP+ x 2; Infiniband 56G QSFP+ x 2. iSCSI 10G (RJ45) x 2; iSCSI 1G x 4. iSCSI 10G SFP+ x 2; FC16G x 2. iSCSI 10G SFP+ x 2; FC16G x 4. iSCSI 10G SFP+ x 2; Converged host board (4 in 1). iSCSI 1G x 4; FC16G x 2. iSCSI 1G x 4; Converged host board (4 in 1). FC16G x 4; iSCSI 10G SFP+ x 2. FC16G x 4; Converged host board (4 in 1). Converged host board (4 in 1); SAS 12G (for JBOD expansion). iSCSI 10G (RJ45) x 2; SAS 12G (for JBOD expansion). iSCSI 10G SFP+ x 2; SAS 12G (for JBOD expansion). SAS 12G x 2; SAS 12G (for JBOD expansion). iSCSI 1G x 4; SAS 12G (for JBOD expansion). FC16G x 2; SAS 12G (for JBOD expansion). FC16G x 4; SAS 12G (for JBOD expansion). iSCSI 40G QSFP+ x 2; SAS 12G (for JBOD expansion). Infiniband 56G QSFP+ x 2; SAS 12G (for JBOD expansion). |  |
| DIMM Configuration Requirements | Default configuration: 4GBx1 Support DDR4 up to 64G (32Gx2) Optional configurations: 8GB (4GBx 2), 16G (8G x2), 32G (16Gx2), 64G (32Gx2) |  |
| Push button requirement | For Reset CTRL to Restore default Status |  |
| LCD and LED Requirements | Only support LED PNL; LED follows ES_LED_FP_v1.22 or later version |  |
| HDD Support requirements | Support mixing SATA 3G/6G drives on AVL Support SATA-SSD (SSD: follow HDD AVL plan) |  |
| Form-factor requirements | Follow Enhance 3US and 2US ME design. |  |
| RoHS requirement | RoHS complaint is a MUST from development stage |  |
| PSU Requirements | For 2U12/3U16 platform: Delta-250W RAID Form-factor; Enclosure Name; Controller; PSU; Vendor. 2U8; 2US; Generic; 250 Watt; Delta. |  |
| Patent Concern | N.A. |  |
| MTBF | Controller: 500,000 hours Subsystem: 25,000 hours Power supplies: 250,000 hours Fans: 500,000 hours |  |
| EMC / Safety | Certification product names: should follow IFT business method of CTO EMC: BSMI, FCC / CE Class B (Class A is Acceptable). Safety: UL / cUL, CB, and ROSTEST (Russia Certificate) |  |
| Env. | - Thermal: System working temperature should follow specs. - Normal Operating: The temperature should be below 40C. - Abnormal Operating (FANx1 or PSUx1 Failure): The temperature should remain below 40C - Due to CPU spec, it cannot operate below 5C. Please test the system above 5C or equal to 5C to meet the CPU spec. - Thermal concern Super Cap the Spec of Super cap is -25C~60C, Therefore it is recommend that w/o Super cap installed the room temperature cannot over 40C and W super cap installed the room temperature cannot over 40C. |  |
| Interoperability Req. | Refer to TD-4-06 Interoperability Test item of later version for SxF |  |

Section 2: User interface and customer related information

| Category | Functional Specification | Remark |
| --- | --- | --- |
| Connectors of rear-side of CTRL | For Single CTRL - 1* mini USB connectors provide COM1 connectivity to HST. - 1* RJ-45 connectors (1 in 1) provide 1GB Management function – on Main board - 4* RJ-45 connectors (1 in 1) provide 1GB Fast Ethernet LAN connectivity – on Main board - 1* USB 3.0 connector – on Main board |  |
| CTRL Drawing |  |  |
| LED indicator on CTRL | Please refer to ES_LED_FP_v1.22 or later version for detailed LED definitions Ctrl 1 LED in green/Amber, for Controller Status Ctrl 2 LED in amber, for Cache Dirty status Steady on; 1. Cache Memory is dirty or 2. Flushing data from FBM to drive (when power back). Blink; Writing cached data from cache memory to FBM* during power outage. Once writing is done, all LEDs are turned off.. Off; No data is in cache memory. Note : FBM : Flash Backup Module Ctrl 3 LED in green, Fast Blink for Host Busy Restore Default Button: LED in GREEN, the default of NVRAM is restored Management Port *1 (1 in 1 connector): Speed LED on RJ-45 in green for 1G and blank for 10/100, Active LED Blink for LAN is active iSCSI 1G *4 ports (1 in 1 connector): Speed LED on 4x RJ-45 in green for 1G and blank for 10/100, Active LED Blink for LAN is active |  |
| Connectors of rear-side of Host Boards of a single CTRL | For iSCSI 1G 4* iSCSI 1G cages provide 1G connectivity to HST For iSCSI 10G (RJ45) 2* iSCSI 10G cages provide 10G/1G/100Mconnectivity to HST For iSCSI 10G (SFP+) 2* iSCSI 10G cages provide 10G connectivity to HST– (2 on each Host board) For SAS 12G (same for SAS 12G for JBOD Expansions) 2* SAS 12G cages provide 12/6/3G connectivity to HST– (2 on each Host board) For FCoE (4 in 1) 4* SFP+ cages provide FC 8G x 4 or FC 16G x 2 or iSCSI 10G SFP+ x 4 connectivity to HST– (4 on each Host board) For FC 16G x 2 2* FC 16G cages provide 16/8/4G connectivity to HST– (2 on each Host board) InfiniBand 56G (FDR) x2 2* InfiniBand 56G connectivity to HST– (2 on each Host board) For FC16G x4 4* SFP+ cages provide FC16G x 4 or FC 8G x 2 or FC4G x 4 connectivity to HST– (4 on each Host board) Ethernet 40G Host Board x2 2* 40G connectivity to HST– (2 on each Host board) |  |
| LED indicator on Optional Host Boards | iSCSI 1G x4 ports Host Board LED 8 x GbE LEDs (two on each GbE port) to indicate the speed/ link status For Speed LED: CH4/CH8; CH5/CH9; CH6/CH10; CH7/CH11. LED on; 1G; 1G; 1G; 1G. LED off; 10/100Mbps; 10/100Mbps; 10/100Mbps; 10/100Mbps. For Link LED: CH4/CH8; CH5/CH9; CH6/CH10; CH7/CH11. Steady on; Link; Link; Link; Link. Blink; Active; Active; Active; Active. Off; No link; No link; No link; No link. ------------------------------------------------------------------------------------------------------------------------ iSCSI 10G (RJ45) x2 ports Host Board LED 4 x GbE LEDs (two on each GbE port) to indicate the speed/ link status For Speed LED: CH4/CH8; CH5/CH9. Green; 10G; 10G. Amber; 1G; 1G. LED off; 100Mbps; 100Mbps. For link status LED: CH4/CH8; CH5/CH9. Steady on; Link; Link. Blink; Active; Active. Off; No link; No link. ----------------------------------------------------------------------------------------------------------------------------- iSCSI 10G (SFP+) Host Board LED 1* CH0 LED in green, for CH0 link status Steady on; Link. Blink; Active. Off; No link. 1* CH1 LED in green, Steady On for CH1 link: spec is same as CH0 LED. ---------------------------------------------------------------------------------------------------------------------------- SAS 12G x2 ports Host Board LED (Same for SAS 12G host board support JBOD expansions) 2* LEDs (one for each host port), to indicate link status For Link status LED: CH4/CH8; CH5/CH9. Steady on; Link; Link. Blink; Active; Active. Off; No link; No link. ---------------------------------------------------------------------------------------------------------------------------- 4 in 1 Host x4ports Board LED 8* LEDs (2 for each host port), to indicate link / speed status 2* Mode LEDs, to indicate mode status For Mode LED: 16G/8G - Green; 16G Fiber mode (only 2 ports can support this mode). 16G/8G - Amber; 8G Fiber mode. 10G - Green; 10G Ethernet mode. For Link status LED: CH4/CH8; CH5/CH9; CH6/CH10; CH7/CH11. Green; Link; Link; Link; Link. Off; No link; No link; No link; No link. For Speed LED (16G Fiber mode): CH4/CH8; CH5/CH9; CH6/CH10; CH7/CH11. Green; 16G; 16G. Amber; 8G; 8G. Off; 4G or no link; 4G or no link. For Speed LED (8G Fiber mode): CH4/CH8; CH5/CH9; CH6/CH10; CH7/CH11. Green; 8G; 8G; 8G; 8G. Amber; 4G; 4G; 4G; 4G. Off; 2G or no link; 2G or no link; 2G or no link; 2G or no link. For Speed LED (10G Ethernet mode): CH4/CH8; CH5/CH9; CH6/CH10; CH7/CH11. Green; 10G; 10G; 10G; 10G. Amber. Off; No link; No link; No link; No link. -------------------------------------------------------------------------------------------------------------------------------- FC 16G x 2 Host Board LED 2* CH0/1 LEDs, to indicate CH0 link / speed status Link status LED: LED in Green for Link, and off for unlink Speed status LED: Green; 16G. Amber; 8G. LED off; 4G. 1* CH0 LED in green, for CH0 link status Steady on; Link. Blink; Active. Off; No link. --------------------------------------------------------------------------------------------------------------------------------- FC16G x 4 Host Board LED Each channel has three LED to indicate different speed as below showed. For 16G Speed LED:. CH0; CH1; CH2; CH3. Green on; Linked. Green Blinking; Data Transfer. LED off; No connect. For 8G Speed LED:. CH0; CH1; CH2; CH3. Green on; Linked. Green Blinking; Data Transfer. LED off; No connect. For 4G Speed LED:. CH0; CH1; CH2; CH3. Amber on; Linked. Amber Blinking; Data Transfer. LED off; No connect. ------------------------------------------------------------------------------------------------------------------------------------------------ Ethernet 40G Host Board LED 2* CH0/1 LEDs, to indicate CH0 link / speed status Link status LED: LED in Green for Link, and off for unlink Speed status LED: Green; Link. Amber; Active. LED off; Not connected. |  |

Section 3：CTRL PCBA and Host Board PCBA requirement

| Category | Functional Specification | Remark |
| --- | --- | --- |
| 2U8: RAID engine G Models CTRL PCBA (3841GSP08E) | For GSe Pro 3008 Models CTRL : For 2U8 serial RAID engines CTRL use the same PCBA (PCBA named as 3841GSP08E) |  |
| RAID engine Host Boards: | For RAID engine Host Boards : - iSCSI 1G 4 ports host boards 9483CE1HIO4 - iSCSI 10G (SFP+) 2 ports host boards 9483CE1HIO2 - iSCSI 10G (RJ45) 2 ports host boards 9483CE2HIO2 - SAS 12G 2 ports host boards 9483CS2HIO2 - 4 in 1, 4 ports host boards 9483CC1HIO4 - FC16G 2 ports host boards 9483CF9HIO2 - FC16G 4 ports host boards 9483CF9HIO4 - Infiniband 2 ports host boards 9483CE3HIO2 - iSCSI 40G 2 ports host boards 9576CE4HIO2 - SAS 12G 2 ports for JBOD Expansions host boards 9483CSJHIO2 |  |
| DRV backplanes | Different products use different Base-units w/ DRV BP PCBAs. And naming are listed as below: F/F; G/R; Base-unit; DRV BP PCB; DRV BP PCBA; Models. 2U8; G; ESS12G2B2Dx-0035; 9583ADP08G; 9583ADP08G; GSe Pro 3008. Note 1. - D means EonStor DS, - x=0 means no bundle HDD and x=P means half qty of bundle HDD (8 for 3U, and 6 for 2U) Please refer to ESS16F-X1840_FTS_V1.0 or later version for detail spec Please refer to \\Server02\pm\PCC infor system\A667_models_naming for Enclosure/PCBA naming |  |
| Other small boards: | - Power board: 9583PWR2U 2U height, redundant PSU design. - Adaptor board: 9583ADP To access Power board and Backplane. |  |
| Cache Backup Module for Caching Data | R CTRLs are having Cache Backup Module for default. G CTRLs are option. |  |
| FW feature spec | Use FW v3.91 or later version (follow EonStor G6) |  |
| SW feature spec | Use SW v2.3.x or later version (follow EonStor G6) |  |

Appendix 1: CTRL Key parts and general information of each model

| Controller | DS RAID CTRL |
| --- | --- |
| 3841GSP08E | GSe Pro 3008 |
| Host Boards | DS RAID Host Board |
| iSCSI 1G 4 ports host boards 9483CE1HIO4 | GSe Pro 3008 |
| iSCSI 10G (SFP+) 2 ports host boards 9483CE1HIO2 |  |
| iSCSI 10G (RJ45) 2 ports host boards 9483CE2HIO2 |  |
| SAS 12G 2 ports host boards 9483CS2HIO2 |  |
| 4 in 1, 4 ports host boards 9483CC1HIO4 |  |
| FC16G 2 ports host boards 9483CF9HIO2 |  |
| FC16G 4 ports host boards 9483CF9HIO4 |  |
| Infiniband 2 ports host boards 9483CE3HIO2 |  |
| iSCSI 40G 2 ports host boards 9576CE4HIO2 |  |
| SAS 12G 2 ports host boards 9483CSJHIO2 |  |
| Memory module per CTRL |  |
| DDR4 RDIMM 4GB | Default, 4GB*1 |
| DDR4 RDIMM 8GB | Optional, 4GB*2 |
| DDR4 RDIMM 16GB | Optional, 8GB*2 |
| DDR4 RDIMM 32GB | Optional, 16GB*2 |
| DDR4 RDIMM 64GB | Optional maximum, 32GB*2 |

Appendix 2: Spare parts and Accessories

| CTRL FRU P/N | GSe Pro 3008 |
| --- | --- |
| 3841GSP08E | V |

| GSe Pro 3000 Series |  |  |  |  |
| --- | --- | --- | --- | --- |
| BP FRU P/N |  | GSe Pro 3008 |  |  |
| 9583ADP08G |  | V |  |  |
| Host Board FRU P/N |  | GSe Pro 3008 |  |  |
| iSCSI 10G SFP+ (RES10G1HIO2-0010) |  | Y |  |  |
| iSCSI 10G RJ45 (RES10G1HIO2-0010) |  | Y |  |  |
| SAS12G (RSS12G1HIO2-0010) |  | Y |  |  |
| 4 in 1 (RFE16G1HIO4-0010) |  | Y |  |  |
| iSCSI 1G RJ45 (RER01G1HIO4-0010) |  | Y |  |  |
| FC16G x 2 (RFC16G1HIO2-0010) |  | Y |  |  |
| FC16G x 4 (RFC16G1HIO4-0010) |  | Y |  |  |
| iSCSI 40G x 2 (RER40G1HIO2-0010) |  | Y |  |  |
| SAS12G (RSS12J1HIO2-0010) |  | Y |  |  |
| Level-1 | Level-2 |  | FRU P/N | GSe Pro 3008 |
| Cables | FC |  | 9270CFCCab04-0010 | Y |
|  |  |  | 9270CFCCab05-0010 | Y |
|  |  |  | 9270CFCCab06-0010 | Y |
|  | Mini SAS 4X(SFF-8088) to Mini SAS 4X(SFF-8088) |  | 9270CmSASCab1-0030 (0.5M) | Y |
|  |  |  | 9270CmSASCab2-0030 (1.2M) | Y |
|  |  |  | 9270CmSASCab6-0030 (1.7M) | Y |
|  |  |  | 9270CmSASCab7-0030 (2.6M) | Y |
|  | SAS 12G Cables |  | 9370CmSASCab1-0030 (0.5M) | Y |
|  |  |  | 9370CmSASCab2-0030 (1.2M) | Y |
|  |  |  | 9370CmSASCab3-0030 (2.6M) | Y |
|  | Mini USB Cable |  | 9370ASCAB-0010 | Y |
| SFP module | 8G |  | 9370CSFP8G-0010 | Y |
|  | 10G |  | 9370CSFP10G-0010 | Y |
|  | 16G |  | 9370CSFP16G-0010 | Y |

### 2. Enclosure FRU

| Level-1 | FRU P/N | GSe Pro 3008 |
| --- | --- | --- |
| PSU | 9583CPSU-0010 | Y |
| Slide Rail | 9571CSLIDER-0010 | Y |
| Ear Rail | 9571CEAR-0010 | Y |

### Appendix 5: Content of Accessory Box (for ref.)

For GSe Pro 3008 series models

- One GUI CD pack, HW manual and operation manual inside

- One EonOne CD pack

- Two power cords

- One mini-USB to DB-9 cable

- 40 screws for HDD installation

- 16 screws for mounting enclosure to rack（M6 x4 and M5 x4 and #10-32 x4）

| Models | GSe Pro 3008 series + Optional Host Boards |
| --- | --- |
| CPU | Intel CPU Broadwell-DE D-1517 (3841GSP08E) |
| RAID ENGINE | CPU/SW RAID |
| Memory socket | DDR4 x 2 sockets |
| DIMMs | 4GB RDIMM x 1 (default) |
|  | (MAX 64GB per CTRL) |
| Flash ROM | 16M x 2 |
| NVRAM | FRAM FM21L16 (2Mbit) |
| Flash Backup Module | m.2 64G MLC Flash |
| Configuration LAN chip | Broadwell-DE + Intel i218 |
| Configuration LAN port | RJ45 w/ 2 LED x 1 (phy) |
| On Board LAN (1G) | RJ45 w/ 2 LED x 4 (i210) |
| M.2 connector | Yes x 1 |
| RS232 port | min USB |
| USB 3.0 | x1 |
| LED indicator | - Controller status |
|  | - CBM status |
|  | - Cache Dirty Status |
|  | - Host busy |
|  | - LAN Link |
|  | - LAN Active |
|  | - Restore Def |
|  | - 4*CH0 Link / Speed status (iSCSI 1G) |
|  | - 4*CH1 Link / Speed status (iSCSI 1G) |
| Push button | Yes (HW reserve only) |
| LCD/ LED PNL | LED support only |
| Buzzer | Yes |
| Driver Interface | SATA only |
| Drive chip | (1) Marvell 9235*1 |
|  | (2) Integrated SATA 6G port |
| Drive channel allocation | Please check the Block Diagram for details |
| Connector to BP | (1 x 5*6 pin connector + 2 x 9*6 pin “4 wall” Airmax) |
| I2C interface | YES |
| PCBA | 3841GSP08E + Optional Host Boards |

Architecture for system block diagram

For GSe Pro 3008 series

ME Drawing

- Front view

- Rear view

- System

- CTRL