A&E Specifications Rev 101411 CAM2301

CAM2301 Megapixel Color Camera

Bid Specification

1.0 Description

CAM2301 is a professional IP network camera utilizing Internet Protocol (IP) to transmit video streams and control signals over networks. Capable of operating over both LANs and WANs, CAM2301 provides a complete budget-conscious remote surveillance solution that is ultra clear and highly integrated. The CAM2301 combines a user-friendly interface and simplified installation with a powerful feature set such as Motion Detection to provide users an easy upgrade path to new digital surveillance system in a virtual environment. These highlights make CAM2301 an ideal choice for environments that require remote surveillance or video transmission.

2.0 Bid Specification

- The camera shall utilize a high sensitivity 2 Megapixel CMOS sensor with 1/2.7”optical format.

- The camera shall have user-selectable lens, compatible with C/CS format lenses.

- The camera shall have Day/Night ICR capabilities.

- The camera shall have a box enclosure.

- The camera shall be H.264 (MPEG4, Part 10) compliant.

- The camera shall have triple standard compression support with simultaneous streaming of H.264, MPEG-4, and MJPEG formats.

- The camera shall have multi-streaming support of up to 2 non-identical concurrent streams (different frame rate, bit rate, resolution, quality, and compression format).

- The camera shall have bit rate control for full view, multi view, and archiving.

- The camera’s bit rate control shall be selectable from 32 Kbps to 10 Mbps for each independent stream.

- The camera shall have Real Time Streaming Protocol (RTSP) support allowing for compatibility with media players such as RealPlayer, Apple QuickTime, VLC Player and others.

- The camera’s H.264 implementation shall maintain full real time video frame rates.

- The camera shall output at a maximum resolution of 1920(H) x 1080(V) pixels at a maximum frame rate of 15 frames per second (FPS).

- It shall be possible to program the camera to output a variety of lower resolution images. For example, 1280(H) x 1024(V) pixels at 30FPS, 1280(H) x 720(V) pixels at 30FPS, or 640(H) x 480(V) at 30FPS.

- The camera shall be equipped with a 100 Mbps LAN connector.

- The camera shall support a minimum HTTP, and RTSP/RTP network protocols.

- The camera shall feature precise motion detection with a minimum of 3 detection zones.

- The camera shall feature automatic exposure, automatic white balance, shutter speed control, programmable brightness, sharpness, instantaneous digital zoom, and electronic image mirror and flip.

- The camera shall support a minimum illumination of 0.5 Lux.

- The camera’s primary power source shall be Power over Ethernet (PoE) complying with the IEEE 802.3af standard or 12V DC power supply.

- The camera’s operating ambient temperature shall be -10~50°C (14~122°F) for warm start conditions.

- The camera shall be FCC Part 15, Class A and CE compliant.

- The camera shall have total unit dimensions of 74.95mm x 59.3mm x 153.5mm (2.95" x 2.34" x 6.04").

- The camera shall have a total unit weight of: Net: 435g (0.96 lb).

Camera Specifications

3.0 Minimum Performance Specification

Megapixel camera must meet the following operating requirements

Imaging

- 2 megapixel CMOS image sensor

- 1920x1080 pixel array

- 1/2.7” optical format

- Bayer mosaic RGB filter

- Minimum illumination of: 0.5 Lux @ F1.0

Full Field of View (FOV) Resolutions

- 1920x1080

Cropped Field of View Resolutions

- 1280x1024 SXGA

- 1280x720 HDTV-720p

- 640x480 VGA

- 320x240 SIF

Data Transmission

- Data rate

  - Bit rate control available for full view, multi view, and archive

  - Bit rate control from 32 Kbps to 10 Mbps

- Video frame rate up to 25fps @ 1920x1080

- Compression type

- H.264 (MPEG4, Part 10)

- Motion JPEG

- MPEG-4

  - HTTP, RTSP image transmission protocols

  - 100Base-T Ethernet Network Interface

Programmability

- Auto Exposure (AE) and Gain Control (AGC)

- Programmable backlight compensation

- Auto white balance

- Electronic image flip - 180 degree rotation

- Programmable shutter speed to help control motion blur

- Proprietary noise cancellation

- Programmable resolution, brightness, saturation and sharpness

- Bandwidth and storage savings by running at 1/4 resolution

Electrical

- Power over Ethernet (PoE): PoE 802.3af

- DC input: auxiliary 12 V DC

- Power consumption 7 Watts maximum

Regulatory Approvals

- FCC, Class A

- CE and RoHS compliant

Environmental

- Operating temperature: -10~50°C (14~122°F)

- Storage temperature: -30~60°C (-22~140°F)

- Humidity: 5~90% (non condensing)

Mechanical

- Die-cast aluminum chassis

- Total unit dimensions: 74.95mm x 59.3mm x 153.5mm (2.95" x 2.34" x 6.04")

- 435g (0.96 lb)

Lens

- Changeable C/CS mount

Related Documentation

- CAM2xxx Series User Manual

- Network Camera SDK HTTP/RTSP, ActiveX and DLL control editions

4.0 Model Numbers

The camera shall be Surveon model CAM2301, 2 megapixel network camera.

5.0 Warranty

Minimum 3 year parts and labor